How to flash essentielb DMI & LOGO on

Pegatron devices

Tools:

Get Project ID:

oemmaker /getprojectID=A35

Sample Project IDs:

C17A project ID is C77881AA
H90T project ID is 3CF6253F

C15M project ID is FCF8AF03

C15a project ID is F54EE328

D17D projectID is 2AC54D9C

D17s project ID is a916c85b

Logo update process:

Please input "logo.bat project_ID"

C17A sample DMI table:

| Item | SMBIOS Structure | Type | Offset | Reference |
| --- | --- | --- | --- | --- |
| 1 | System Information | Type1 | 04 | essentielb |
| 2 |  | Type1 | 05 | Smart MOUV series |
| 3 |  | Type1 | 19 | Smart MOUV 1705 |
| 4 |  | Type 1 | 1A | essentielb |
| 5 | Base Board | Type2 | 04 | essentielb |
| 6 |  | Type 2 | 05 | Smart MOUV series |
| 7 | Enclosure or Chassis | Type3 | 04 | essentielb |

Boot from DOS USB-Stick:

Write DMI:

su.exe -EXTF -type 1 /wdmi 04 essentielb

su.exe -EXTF -type 1 /wdmi 05 "Smart MOUV series"

su.exe -EXTF -type 1 /wdmi 19 "Smart MOUV 1004"

su.exe -EXTF -type 1 /wdmi 1A essentielb

su.exe -EXTF -type 2 /wdmi 04 essentielb

su.exe -EXTF -type 2 /wdmi 05 "Smart MOUV series"

su.exe -EXTF -type 3 /wdmi 04 essentielb

Read DMI:

For checking DMI type 1: su –type 01 /rdmi

For checking DMI type 1: su –type 01 /rdmi

For checking DMI type 1: su –type 01 /rdmi

- "Windows_DMI_commands" is to flash DMI commands under Windows. Only use if DOS DMI-Tools failed to flash.

- "reset_DMI" is to set the default values into BIOS. Use only, if you flashed something to wrong Offset.